
WorldTracker GPRS
[image: image18.png]$GPRMC (out) — UTC, position, course, speed, etc.

Example
$GPRMC | ,123456.789|,A | ,3444.1234N | ,13521.4567E ,005.6... ...|,123.52| ,020197
Field# 1 2 3 4 5 6 7 8 9
5 |*07| GRELF
10 11 12,
#. Description Range
1. UTC: Time
“127:hh 00-23
“34”: mm 00-59
“567: 58 00-59
". 789" : sss .000 - .999
2. Status AorV
“A” Data valid
(Stand-alone or DGPS)

“V”: Navigation receiver warning
3-4. Latitude

“34”: degree 00-90
“44”: minute (integer) 00-59
“1234”: minute (fraction) 0000-9999
“N”: North/South NorS

5-6 Longitude

“135”: degree 000-180
“21”: Minute (integer) 00-59
“4567”: Minute (fraction) 0000-9999
“E”: East/West Eor W

7. Speed (kts) 000.0-999.9

Note: A null field is output unless speed information is available.

8. True Course (degree)
42352 000.00-359.99
Note: A null field is output unless true course information is available.

9. UTC: Date

“02”: DD 01-31
“017: MM 01-12
97 YY 97-40

(1997-2040)

10 Magnetic Deviation (degree)

& w SiRF Technology Inc. does not support magnetic declination.

WorE

“W”: West (MAG = TRUE-DEV)

“B”: Bast (MAG = TRUE+DEV)

12. Checksum

8 bits data between “$” and “*” (excluding “$” and “*”) are XORed, and the result is converted to 2
bytes of hexadecimal letters. Only RMC sentences are transmitted with checksum. All other output

sentences do not include checksum fields.





Table of Contents
1. Hardware Description………………………………………………………………………….4

1.1 Front face…………………………………………………..………………………………..……….….…….……………………....4

1.2 Side face…………………………………………………………………………………………….……….……...…………………….4

1.3 Bottom face………………………………………………………………………..……………….….………..…….…………………5

1.4 Charging…………………………………………………………………………………………………………………..………………..5
1.5 Rear face……………………………………………………………………………………………………………..……………………..6
2. Specifications……………………………………………………………………………………….7

3. Function Overview…………………………………………………..…………………..………8

3.1 General…………….…………………………………………..………………………………………..……………….……………………8

3.2 Features………………………………………………………………..……….…………………………………..…………………………9

3.3 Applications………………………………………………………………………….………………………..……………………………..9

4. Easy Start…………………………………………………..…………..……………..……………10

5. Setup..………………………………………………………………………..……………….………12

5.1 Set up the username……………………………………...………………..……………………………………………...………..13

5.2 Change password………………………………………………………………..……………………………..……………………....14

5.3 Set up the GPRS APN (Access Point Name)……………………………...…………..……………………………………15

5.4 Set up a host name to transmit data……………………………………………..……...…………………………….......16

5.5 Set up the phone number list……………………………………………………………..…………..……………………………17

5.6 Set up the Auto Report………………………………...………………………………………..…………………….………………18

5.7 All parameters reset to default…………………………………………………………………..…………………….………....19

5.8 Acquire the report of current position……………………………………………………………..…………….……………..20

5.9 Set up an E-mail address to receive the data…………..……………………………………………………………….….21

5.10 Set up the SMTP server……...……………………………………..………………………………………….………..………….22

5.11 Set up the router for transmitting date either by URL/IP or E-mail…….…………….…….…..………….23

5.12 Request Imei code….………………………………………………………………………………….……………………………....25

5.13 Deactivate Panic mode……………………………………………..…………………………………..…………………………….26

6. Respond messages…………………………………………………………………..………...27

6.1 Auto response………………………………………………………………………..…………………………..……………….……….28

6.2 Emergency response………………………………………………………….………………………………………………………...29

  6.3 Polling response……………………………………………………………………………………………………….…………………….30

6.4 Low power response……………………………………….………………………………………………………………………...31

6.5 NMEA 0183 GPRMC Sentence…………………………………………………………………………..………………….……32

7.Warranty………………………….….…….………………………………………………….……33

1 Hardware Description

1.1 Front face

[image: image1.jpg]


[image: image17.png]$GPRMC (out) — UTC, position, course, speed, etc.

Example
$GPRMC | ,123456.789|,A | ,3444.1234N | ,13521.4567E ,005.6... ...|,123.52| ,020197
Field# 1 2 3 4 5 6 7 8 9
5 |*07| GRELF
10 11 12,
#. Description Range
1. UTC: Time
“127:hh 00-23
“34”: mm 00-59
“567: 58 00-59
". 789" : sss .000 - .999
2. Status AorV
“A” Data valid
(Stand-alone or DGPS)

“V”: Navigation receiver warning
3-4. Latitude

“34”: degree 00-90
“44”: minute (integer) 00-59
“1234”: minute (fraction) 0000-9999
“N”: North/South NorS

5-6 Longitude

“135”: degree 000-180
“21”: Minute (integer) 00-59
“4567”: Minute (fraction) 0000-9999
“E”: East/West Eor W

7. Speed (kts) 000.0-999.9

Note: A null field is output unless speed information is available.

8. True Course (degree)
42352 000.00-359.99
Note: A null field is output unless true course information is available.

9. UTC: Date

“02”: DD 01-31
“017: MM 01-12
97 YY 97-40

(1997-2040)

10 Magnetic Deviation (degree)

& w SiRF Technology Inc. does not support magnetic declination.

WorE

“W”: West (MAG = TRUE-DEV)

“B”: Bast (MAG = TRUE+DEV)

12. Checksum

8 bits data between “$” and “*” (excluding “$” and “*”) are XORed, and the result is converted to 2
bytes of hexadecimal letters. Only RMC sentences are transmitted with checksum. All other output

sentences do not include checksum fields.




[image: image2.jpg]



1.2 Side Face

[image: image3.jpg]



1.3 Bottom Face

[image: image4.jpg]



[image: image5.jpg]



1.4 Charging

	[image: image6.jpg]



	[image: image7.jpg]





1.5 Rear Face (Battery Cap)
[image: image8.jpg]



	[image: image9.jpg]



	[image: image10.jpg]



	[image: image11.jpg]




	Use your thumb to press the battery cap and then slide downward to open it.
	Open the cap and move remove the battery.
	Insert in the SIM card .


2 Specifications
[image: image12.png]GPS Receiver

General
GPS frequency L1,1575.42MHz
C/A code 1.023MHz chip rate
Datum WGS-84
Sensitivity -159 db
Channels 20 channel all-in-view tracking
Acquisition Time
Reacquisition <0.1 sec
Cold <42 sec TTFF (Time To First Fix)
Warm <38 sec TTFF
Hot <1 sec TTFF
GSM Transceiver

Frequency Tri-band 850/1800/1900 MHz

ICompliant to GSM Phase 2/2+
GPRS Transmission IGPRS multi-slot class 10

IGPRS mobile station class B

General

/Antenna Internal GSM antenna and active GPS patch
lantenna

External Power Supply 5.0 ~ 8.0V DC input charging current 500mA

Battery Power Supply 3.7V DC

Rechargeable Li-ion Batteries  |Standard: 1100mA
Optional: 1800mA

Battery Life 22 hours after full charged when sending one
report every 60 minutes

Device Size 64.66mm(L) x 43.19mm(W) x22.81mm (H)

Device Weight 85g

Environmental Operation Temperature | -5~ +60 C

Storage Temperature -5 ~+60C





3 Function Overview

3.1 General

WorldTracker  GPRS is using the GPRS service to upload the RMC sentences containing latitude, longitude, speed and time to an assigned web server. 

	Note1：When GPRS network is not available, SMS will be sent instead in the HTTP/SMS mode.


3.2 Features
● Panic
The diagram below shows the result after Panic button is pressed under HTTP/SMS mode.










● Auto Report
You can set up auto report function to receive a regular report, and note that the default setup is off, Please refer to chapter 5.9 for detailed information.
3.2 Applications：

[image: image13.png]



Fleet Management 

[image: image14.png]



Vehicle Security and Recovery

[image: image15.png]



Asset tracking

[image: image16.png]



Covert Tracking

4. Easy Start

All the settings are by SMS. Please use your cellular phone to send the following SMS to WORLDTRACKER GPRS to start experiencing it.

1. Request for the imei code of WORLDTRACKER GPRS. 

If the command has been transferred successfully, WORLDTRACKER GPRS will send an imei code to your cell phone otherwise a failure message will be sent instead.

＃Username，0000，Imei＊
2. Set up APN (Access Point Name) that is provided by your GSM provider. 

If you are not sure about your APN, please refer to the APN table in the CD or contact your local mobile phone service. After sending the SMS command, WORLDTRACKER GPRS will response a message to your cell phone to see if the WORLDTRACKER GPRS does receive the command. Please note that the WORLDTRACKER GPRS is not able to determine the validity of the APN.
＃Username，0000，3，your APN，user，password＊
3. Set up the phone book. 

When either APN or URL is incorrect or GPRS/URL is not available, the WORLDTRACKER GPRS will not be able to upload the data to the website. In any case mentioned above, the WORLDTRACKER GPRS will send a SMS instead containing GPS position to the 1st predefined number.

＃Username,0000,5,cellular phone number*
4. Set up Auto Report.
After sending the SMS command shown below with success, the WORLDTRACKER GPRS will start sending data to Security Concepts’s Website at the interval of 60 seconds and total of 5 reports. However, if either GPRS is not available or APN is incorrect or URL is down, the WORLDTRACKER GPRS will send the data via SMS to the 1st predefined number in the phone book.

＃Username，0000，6，60，5＊

5 
Setup
WORLDTRACKER GPRS currently provides 13 commands for setup. You can setup the WORLDTRACKER GPRS by following the instructions shown in this chapter via SMS from a cellular phone. Key the specific SMS message in your cellular phone as the instruction and send it to the SIM number of WORLDTRACKER GPRS. The setup messages are showed in the following sections.

	Note1: The default settings are shown as below.

a. Default WORLDTRACKER GPRS ID is “Username”, and please read Chapter 5.1 in order to change the ID.

b. Default password is “0000”, and please read Chapter 5.2 in order to change the password.

c. There’s no number in the default contact, and please read Chapter 5.5 in order to add the phone number.

d. Default setting of Auto Report is deactivated, and please read Chapter 5.6 in order to setup the auto regular report.

Note2: Before doing any setup, please ensure WORLDTRACKER GPRS is connected with the GSM network. The red LED must lights on. And please note that WORLDTRACKER GPRS must not be under Panic mode.

Note3: While you are keying in any setup message, note that NO space is allowed between the characters in the SMS. Characters can be letters, commas or any signs.

Note4: Any setup message must be started with a “#” sign and ended up with a “*” sign.

Note5: Only when the password, username and setup message are correct, the WORLDTRACKER GPRS will update information according to user’s definition. If the command is not valid, a failure report is going to be sent to the commander cell phone. 


5.1 Change Username

You can change the ID of WORLDTRACKER GPRS by following the format below. No space is allowed between the characters. For example, if you send the SMS message shown in the example below to the WORLDTRACKER GPRS, you will change the ID from “Username” to “ason”
Setup format： ＃「username」，「Password」，「Function Code」，「New username」＊

Example：＃username，0000，1，ason＊

The table 5.1.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	Default ID of WORLDTRACKER GPRS

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	1
	Mode 1 defines the ID setup

	Ason
	· New ID defined by the owner. 

· At the maximum of 8 letters.

	*
	End sign.


Table 5.1.1

The table 5.1.2 shows each confirmation message reply after setup

	Situation
	Message Reply

	Setup Succeeds
	[username] + WORLDTRACKER GPRS username is updated. 

	Setup Fail
	[username] + WORLDTRACKER GPRS username setup fail!

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, changing username is not allowed.


Table 5.1.2

5.2 Change Password

Use this SMS message example seen below to change the password of WORLDTRACKER GPRS. For example, by following the example below, you will change the default password, “0000” to a new password, “1111”.

Setup format：＃「username」，「Password」，「Function Code」，「new Password」，「new Password」＊

Example：＃username，0000，2，1111，1111＊

The table 5.2.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	2
	Mode 2 defines to change the password

	1111
	New password 

	1111
	Reconfirm the password

	*
	End sign.


Table5.2.1
The table 5.2.2 shows each confirmation message reply after setup

	Situation
	Message Reply

	Setup Succeeds
	[username] + WORLDTRACKER GPRS Password is updated.

	Setup Fail
	[username] + Password setup Fail!

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or SMS format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, changing password is not allowed.


Table 5.2.2

5.3 Set up the GPRS APN (Access Point Name) 

For uploading the data to a web server, you must have a set of GPRS APN from your GSM network. Various GSM network provides different GPRS APN settings. Please check the referential tables for APN in the Chapter 8 before setup. If you could not find the name of your GSM service provider, please ask your GMS service provider about the APN settings. If the username and password of APN are indefinite or showed in blank, just leave the parameters blank (no space). You can follow the Setup Format below to setup the APN that is provided by your GSM network.

Setup format：＃「username」，「Password」，「Function Code」，「access point name」，「user」，「password」＊

Example：＃username，0000，3，internet，user，password＊

  The table 5.3.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	3
	Mode 3 defines the APN 

	Internet
	Name of APN 

	User
	Username of APN

	Password
	Password of APN

	*
	End sign.


Table 5.3.1

The table 5.3.2 shows each confirmation message reply after setup.

	Situation
	Message Reply

	Setup Succeeds
	[username] + WORLDTRACKER GPRS GPRS APN is updated.

	Setup Fail
	[username] + WORLDTRACKER GPRS GPRS APN setup Fail!

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, GPRS APN setup is not allowed.


Table5.3.2

5.4 Set up a host name (URL) to transmit data 

This command (SMS) instructs the WORLDTRACKER GPRS to access an URL or fixed IP for data transmission. Please note that the question mark “?” or any symbol is not allowed to be keyed in behind the URL. The WORLDTRACKER GPRS will add the necessary symbol(s) on automatically.

Setup format：＃「username」，「Password」，「Function Code」，「Fixed IP or URL」＊

Example：＃username，0000，4，http://www.SecurityConcepts.com /gprs.aspx＊

      The table 5.4.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	4
	Mode 4 defines to trasmit data to a specific website

	http://www,Security Concepts.com/gprs.aspx
	Default domain name, Sign “?” is not allowed to write in the area.

	*
	End sign.


Table 5.4.1

The table 5.4.2 is showing each confirmation message reply after setup.

	Situation
	Message Reply

	Setup Succeeds
	[username] + WORLDTRACKER GPRS IP/Domain is updated.

	Setup Fail
	[username] + IP/Domain setup Fail!

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, IP/Domain setup is not allowed.


Table 5.4.2

5.5 Set up the phone number list 

Use this SMS message to predefine the cellular numbers (Max. of 5 numbers) by following the format below.

Setup format: #「username」，「Password」，「Function Code」，「PH-01」，「PH-02」，「PH-03」* 

Example：＃username,0000,5,+886123456789,+492234567890,+866323456789*

Note that both adding and not adding the “+ “sign in front of the cellular phone number(s) are both acceptable, while the”+” sign should precede the national code.

.

The table 5.5.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	5
	Mode 5 defines the changes of the built-in phone number list.

	+886123456789
	1st cellular numbers, PH-01

	+492234567890
	2nd cellular numbers, PH-02

	+866323456789
	3rd cellular numbers, PH-03

	*
	End sign.


Table 5.5.1

The table 5.5.2 shows each confirmation message reply after setup.

	Situation
	Message Reply

	Setup Succeeds
	[username] + WORLDTRACKER GPRS phone book is updated.

	Setup Fail
	[username] + WORLDTRACKER GPRS phone book setup Fail!

	Non-numeric data is input for numbers
	[username] + Parameter error

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, phone book setup is not allowed.


Table 5.5.2
	Note: The program will overwrite the previous contact list if the users do this setup.


5.6 Set up the Auto Report

Use this command to set up the frequency of Auto Report. If you send the SMS according to the example shown below to WORLDTRACKER GPRS, you will have an Auto Report Message every 300 seconds for 99 messages. Once the loop is done, you can send a SMS message to update/renew, or switching off it in order to renew it.

	Note1: You may limit the amount of the Auto reply messages by inputting the digits from 1 to 9998 in the 「times」 field. When you input 9999, the Auto Report Message will not stop unless you define a new Auto Report Setting. 


Setup format：＃「username」，「Password」，「Function Code」，「intervals (sec)」，「total number of report」＊

Example1：＃username，0000，6，300，99＊

The table 5.6.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	6
	Mode 6 defines to change the intervals and times of sending-back data

	300
	A constant interval of sending data

	99
	The amount of messages sent automatically 

	*
	End sign.


Table 5.6.1

The table 5.6.2 shows each confirmation message reply after setup

	Situation
	Message Reply

	Setup Succeeds
	[username] + Setup OK.WORLDTRACKER GPRS Auto Report setting is updated.

	Setup Fail
	[username] + Auto Report setting Setup Fail!

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, Auto Report setting is not allowed.


Table 5.6.2

5.7 All parameters reset to default

  Use this SMS message to reset WORLDTRACKER GPRS to factory default. You can just send the SMS according to the example seen below. It will overwrite all the current parameters to default.

Setup format：＃「username」，「Password」，「Function Code」＊

  Example：＃username，0000，9＊

  The table 5.7.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	9
	Mode 9 defines to change all the parameter to default value

	*
	End sign.


Table 5.7.1

	Situation
	Message Reply

	Setup Succeeds
	[username] + Setup OK. WORLDTRACKER GPRS resets to default

	Setup Fail
	[username] + Reset Fail!

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, reset to default is not allowed.


The table 5.7.2 shows each confirm message reply after setup.

Table 5.7.2
5.8 Acquire the report of current position

You can send the SMS according to the example shown below. It will transmit current data to the web server (URL) at once.

Setup format：＃「username」，「Password」，「Function Code」＊

  Example：＃username，0000，10＊

The table 5.8.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	10
	Mode 10 defines to transmit current data to the web server at once.

	*
	End sign.


Table 5.8.1

	Situation
	Message Reply

	Setup Succeeds
	[username] + Polling OK!

	Setup Fail
	[username] + Polling Fail!

	In mail mode, the setup parameters are empty
	[username] + Mail address or SMTP server parameter is empty

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error


The table 5.8.2 shows each confirm message reply after setup.

Table 5.8.2
	Note that the message reply shown in Table 5.8.2 will only be sent back to cell phone that sent the SMS to do configuration.  The caller’s ID of cellular phone must be activated in order to let the WORLDTRACKER GPRS recognize the caller ID and response accordingly.


5.9 Set up an E-mail address to receive the data

In addition to receiving data from URL, you can also obtain data from a specified E-mail address via GPRS service. After setting up the APN, you can also define an E-mail address to receive data by following the instruction below, please note that, since processing the transformation from APN to E-mail address might cost longer time, so receiving data from URL is still recommended.

Setup format：＃「username」，「Password」，「Function Code」，「Email address」＊

Example：＃username，0000，12，mail@Security Concepts.com＊

The table 5.9.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	· username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	12
	Mode 12 defines to setup an E-mail address data to a specific website

	mail@Security Concepts.com
	The E-mail address that receives the data. 

	*
	End sign.


Table 5.9.1

The table 5.9.2 shows each confirmation message reply after setup. 

	Situation
	Message Reply

	Setup Succeeds
	[username] + mail address Setup OK.

	Setup Fail
	[username] + mail address setup fail.

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, Mail address setting is not allowed.

	Incorrect password 
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[Username] + in emergency, Mail address setting is not allowed.


Table 5.9.2

5.10 Set up the SMTP server

After setting up the E-mail address, you still have to setup a SMTP server so that the WORLDTRACKER GPRS can use the SMTP server send E-mails. Setup SMTP by following instruction:

Setup format：＃「username」，「Password」，「Function Code」，「SMTP server」＊

Example：＃username，0000，13，Security Concepts.com＊

The table 5.10.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	13
	Mode 13 defines to setup a SMTP server

	Security Concepts.com


	The SMTP server 

	*
	End sign.


Table 5.10.1

The table 5.10.2 shows each confirmation message reply after setup.

	Situation
	Message Reply

	Setup Succeeds
	[username] + SMTP Setup OK.

	Setup Fail
	[username] + SMTP server updated fail!

	
	[username] + SMTP server error.

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, SMTP server setup not allowed.

	Incorrect password 
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error

	WORLDTRACKER GPRS is in Emergency Mode
	[Username] +  in emergency, SMTP server setting is not allowed.


Table 5.10.2

5.11 Set up the router for transmitting data either by URL/IP or E-mail

You can choose the router to transmit the data by URL/IP, E-mail or SMS message,. Please refer to the following table for setup the router.

The table 5.11.1 describes the meaning of each segment in the message above.
	Router code
	Representation

	0
	HTTP/SMS, send data to URL, if the URL is untouchable, then send it via SMS instead.

	1
	Mail/SMS, send data to e-mail address, if the e-mail address is untouchable, then send it via SMS instead.

	2
	HTTP only, send data to URL, if the URL is untouchable, it won’t send the data by SMS.

	3
	E-mail only, send data to the e-mail address, if the e0mail address is untouchable, it’s won’t send the data by SMS.

	4
	SMS only, send data by SMS only.


Setup format：＃「username」，「Password」，「Function Code」，「0, 1,2,3 or 4」＊

Example：＃username，0000，14，1＊

The table 5.11.2 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	14
	Mode 14 defines to setup router. 

	0/1/2/3/4


	Please see the definition of the code above

	*
	End sign.


Table 5.11.2

The table 5.11.2 shows each confirmation message reply after setup.

	Situation
	Message Reply

	Setup Succeeds
	[username] + is switching to HTTP/SMS mode

	
	[username] + is switching to Mail/SMS mode

	
	[username] + is switching to HTTP mode

	
	[username] + is switching to Mail mode

	
	[username] + is switching to SMS mode

	Setup Fail
	[username] + is switching to HTTP/SMS mode without success!

	
	[username] + is switching to Mail/SMS mode without success!

	
	[username] + is switching to HTTP mode without success!

	
	[username] + is switching to Mail mode without success!

	
	[username] + is switching to SMS mode without success!

	WORLDTRACKER GPRS is in Emergency Mode
	[username] + in emergency, switching to HTTP/SMS mode is not allowed.

	
	[username] + in emergency, switching to Mail/SMS mode is not allowed.

	
	[username] + in emergency, switching to HTTP mode is not allowed.

	
	[username] + in emergency, switching to Mail mode is not allowed.

	
	[username] + in emergency, switching to SMS mode is not allowed.

	Incorrect password 
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error


Table 5.11.2

5.12 Request Imei code

Imei stands for international mobile equipment identification. We offer you a command to acquire the Imei number from WORLDTRACKER GPRS via SMS. Actually, every WORLDTRACKER GPRS budget has a different default Imei code. You can send the SMS according to the example shown below to check its imei number so as to register to the web server for tracking.

Setup format：＃「username」，「Password」，「Function Code」＊

Example：＃username，0000，imei＊

The table 5.12.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	imei
	Mode imei is to inquire for international mobile equipment identification of WORLDTRACKER GPRS

	*
	End sign.


Table 5.12.1
The table 5.12.2 shows each confirmation message reply after setup.

	Situation
	Message Reply

	Setup Succeeds
	[Username] + “,[imei]”

	Incorrect password 
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error


Table 5.12.2

5.13 Deactivate Panic mode.

In this function, it helps you to deactivate panic mode forcedly. The SMS command is as following:

Setup format：＃「username」，「Password」，「Function Code」＊

Example：＃username，0000，offpanic＊

The table 5.13.1 describes the meaning of each segment in the message above.

	Text Keyed In SMS
	Description

	#
	Start sign.

	username
	· Default ID of WORLDTRACKER GPRS.

· If you have changed the Username, please use the updated one.

	0000
	· Default password. 

· If you have changed the password, please use the updated one. 

	offpanic
	Mode offpanic is to deactivate the panic status of WORLDTRACKER GPRS

	*
	End sign.


Table 5.13.1

The table 5.13.2 shows each confirmation message reply after setup.

	Situation
	Message Reply

	Setup Succeeds
	[Username] + is forced to deactivate panic status.

	Setup Fail
	[Username] + is not in Panic mode

	Incorrect password
	[Username] + Password setup Fail!

	Incorrect username or command format
	[Username] + command error


Table 5.13.2

6: Respond messages

This chapter shows you how to read and understand the different response messages sent from WORLDTRACKER GPRS. When WORLDTRACKER GPRS is in GPRS/SMS mode it will upload the data to the assigned URL first. However, if GPRS service is not available, it will send the SMS to the predefined phone number instead. In the case of GPRS-only mode, WORLDTRACKER GPRS will only send the data by GPRS no matter there is GPRS service or not. Basically, there’re 7 different situations that trigger the WORLDTRACKER GPRS to response its position (Auto Report, Polling, Emergency, Low Power) and are shown in the following pages.

Event Table:

This event table helps you to know what event triggers the response.

	Event
	Descriptions

	Auto
	When the monitor sends a valid Auto Report command to WORLDTRACKER GPRSS, it will send a report including an “Auto” behind each GPRMC sentence.

	SOS
	When the user presses the Panic button, it will send the report including an “SOS” event behind each GPRMC sentence. No configuration is allowed in SOS mode. The SOS mode will only be released by special commands (Master Reset and Off Panic) and switching it off/on.

	LP
	When WORLDTRACKER GPRSS detects its battery ‘s voltage is lower than certain level, it will send a message to the 1st predefined number. The event is “LP” that means low power. When receive this message, please recharge the wevtrac-4.

	POLL
	When using the SMS to poll the report, the event will be “poll”.


6.1 Auto Report

6.1.1. Uploading Auto data to URL when GPS is fixed

When execute the Auto Report according to the settings with a GPS fixed, WORLDTRACKER GPRSS will upload the data to your URL. The data format will be like the following.

Format:

URL?imei=IMEI,RMC=GPRMC,AUTO

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,A,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,AUTO

6.1.2. Uploading Auto data to URL when GPS is not fixed

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,V,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,AUTO

	Field
	Descriptions

	IMEI
	The username of WORLDTRACKER GPRSS that is a unique number.

	GPRMC
	It is a full sentence of GPRMC including the current Lat/Long, UTC and others.

	AUTO
	This is the event triggers the response.


	!! Note: 1. When in GPRS or e-mail mode and there is no GPRS service, the report will be sent to the 1st predefined number. In the mean while, the “IMEI” will be changed to the “Username” and the “Username” is changeable by using SMS command.

2. The “A” is the GPRMC sentence indicates the GPS status, which is GPS fixed. If the GPS is not fixed, letter “V” will substitute the letter “A”.


6.2 Emergency Response

When there is GPRS, WORLDTRACKER GPRSS will upload the data to the URL/e-mail. If the GPRS is not available, it will send the SMS instead. Please read Chapter 3.1 for how Panic works.

6.2.1. Uploading Emergency data to URL/e-mail when GPS is fixed
When there is GPS fixed, send a single SMS to the built-in numbers with the contents like following:

Format:

URL?imei=IMEI,RMC=GPRMC,SOS

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,A,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,SOS

6.2.2. Uploading Emergency data to URL/e-mail when GPS is not fixed

When there is GPS fixed, send a single SMS to the built-in numbers with the contents like following:

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,V,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,SOS

	Field
	Descriptions

	IMEI
	The username of WORLDTRACKER GPRSS that is configurable.

	$GPRMC
	It is a full sentence of $GPRMC including the Lat/Long, UTC and others.

	SOS
	This is the event triggers the response.


	!! Note: 1. When in GPRS or e-mail mode and there is no GPRS service, the report will be sent to the 1st predefined number. In the mean while, the “IMEI” will be changed to the “Username” and the “Username” is changeable by using SMS command.

2. The “A” is the GPRMC sentence indicates the GPS status, which is GPS fixed. If the GPS is not fixed, letter “V” will substitute the letter “A”.


6.3 Polling Response

6.3.1. Uploading Poll data to URL when GPS is fixed
When send an SMS to poll a report with a GPS fixed, WORLDTRACKER GPRSS will upload a data to your URL. The data format will be like the following.

Format:

URL?imei=IMEI,RMC=GPRMC,POLL

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,A,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,POLL

6.3.2. Uploading Poll data to URL when GPS is not fixed
When send an SMS to poll a report without a GPS fixed, WORLDTRACKER GPRSS will upload the data to your URL. The data format will be like the following.

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,V,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,POLL

	Field
	Descriptions

	IMEI
	The IMEI of WORLDTRACKER GPRSS that is a unique number.

	GPRMC
	It is a full sentence of GPRMC including the previous Lat/Long, UTC and others.

	POLL
	This is the event triggers the response.


	!! Note: 1. When in GPRS or e-mail mode and there is no GPRS service, the report will be sent to the 1st predefined number. In the mean while, the “IMEI” will be changed to the “Username” and the “Username” is changeable by using SMS command.

2. The “A” is the GPRMC sentence indicates the GPS status, which is GPS fixed. If the GPS is not fixed, letter “V” will substitute the letter “A”.


6.4 Low-Power Response

6.4.1. Uploading Low-Power data to URL when GPS is fixed
When the power is low and there is a GPS fixed, WORLDTRACKER GPRSS will upload a data to your URL. The data format will be like the following.

Format:

URL?imei=IMEI,RMC=GPRMC,LP

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,A,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,LP

6.4.2. Uploading Low-Power data to URL when GPS is not fixed
When the power is low and there is not GPS fixed, WORLDTRACKER GPRSS will upload the data to your URL. The data format will be like the following.

Example:

http://www.Security Concepts.com/eric-gga/gprs.aspx?imei=351277000000000&
RMC= GPRMC,095838.000,V,2458.9733,N,12125.6583,E,0.41,79.21,220905,,*30,LP

	Field
	Descriptions

	IMEI
	The IMEI of WORLDTRACKER GPRSS that is a unique number.

	GPRMC
	It is a full sentence of GPRMC including the previous Lat/Long, UTC and others.

	LP
	This is the event triggers the response.


	!! Note: 1. When in GPRS or e-mail mode and there is no GPRS service, the report will be sent to the 1st predefined number. In the mean while, the “IMEI” will be changed to the “Username” and the “Username” is changeable by using SMS command.

2. The “A” is the GPRMC sentence indicates the GPS status, which is GPS fixed. If the GPS is not fixed, letter “V” will substitute the letter “A”.


6.5 NMEA 0183 GPRMC Sentence



7 Warranty
Warranty Time Period and Repair Coverage

SECURITY CONCEPTS, INC. warrants WORLDTRACKER GPRS to be free from all defects and malfunctions in materials and workmanship for a period of 12 months from the original purchase date from Security Concepts or authorized dealers. If the equipment functions improperly during the warranty period, Security Concepts will either repair or replace the unit without charge. Such repair service will include necessary adjustments, remanufacture, and replacements. The product should be returned freight-prepaid by the purchaser within valid warranty period. Notice that you must contact Security Concepts for a RMA (Return Material Authorization) number before returning the goods for repair. 
Telephone assistance will also be provided during the warranty period.

Limitations
This warranty is limited only to the repair or replacement of defective parts confirmed by Security Concepts to be a result of faulty materials or workmanship. Instruments mechanically or physically damaged due to the following conditions are beyond our warranty:

Neglect, misuse or abuse, such as a incorrect testing, installation, or operation.

Place subject in extreme environments beyond the limits of the specifications.

Subjected to disassembling, soldering, alteration, unauthorized repair, and electrical shock by nature.

Any incidental or consequential losses or damages result from the purchase. 

Disaster, accident, using any unauthentic substitutive equipment or loss of any accessory that’s not provided by Security Concepts.
For damages caused under the above conditions, we'll contact you to discuss replacement options.
SECURITY CONCEPTS
261 Palm Ave.

Millbrae, CA 94401

650-692-8100

www.trackingtheworld.com 

SOS Button


Press in an emergency











GSM Status Indicator


Flashing: Searching GSM signals


Light on: GSM fixed





GPS Status Indicator


Flashing: Firmware running


Light off: Searching GPS signals


Light on: GPS fixed








Power Switch











When power is OFF


When WORLDTRACKER GPRS is charged with power switched off, you will see a clear green light here indicating it is charging.





When power is ON


 WorldTracker GPRS is charged with power switched on and GPS is fixed, you willl see a faint green light here. That means it is charging.





Power Input





Wall Charger





No GPRS





Send the SMS to the first number in the phonebook instead





Upload messages to URL via GPRS. (The default report frequency is a message every 60 seconds for 60 times)











Send SMS to 1~3 numbers. The SMS message is “Username, Panic key is pressed. Emergency!!”








Press Panic 





GPRS comes back





� EMBED MSPhotoEd.3  ���








7

_1189862717.unknown

